EMBEDUR  MODULE-3 WIFI ASSIGNMENT

G ADITHYA PRASAD

1.Different 802.11 PHY Layer Standards and Their Characteristics

The PHY (Physical) layer of IEEE 802.11 has evolved significantly from the first Wi-Fi release in 1997 to modern Wi-Fi 6/6E. Each amendment improved data rate, efficiency, modulation, and resistance to interference.

The original 802.11 standard operated in the 2.4 GHz band and supported FHSS and DSSS with a maximum speed of 2 Mbps. This was the foundation of wireless LAN.

802.11b improved DSSS technology and increased speed to 11 Mbps using Barker codes and CCK encoding.

802.11a introduced OFDM and moved to the 5 GHz band, achieving 54 Mbps and reducing interference.

802.11g brought OFDM back to 2.4 GHz while maintaining backward compatibility with 802.11b.

802.11n was a major leap introducing MIMO, channel bonding (40 MHz), and frame aggregation, reaching up to 600 Mbps.

802.11ac further enhanced performance using MU-MIMO, wider channels (80/160 MHz), and 256-QAM.

802.11ax (Wi-Fi 6/6E) introduced OFDMA, 1024-QAM, better efficiency in dense networks, and operation in 6 GHz.

This evolution shows how PHY improvements directly increased throughput and spectrum efficiency.

2) DSSS and FHSS – What They Are and How They Work

Both DSSS and FHSS are spread spectrum techniques used in early Wi-Fi to combat interference and noise.

FHSS (Frequency Hopping Spread Spectrum) divides the frequency band into many small channels. The transmitter rapidly hops between these frequencies in a pseudo-random sequence known to both sender and receiver. If interference occurs in one frequency, the system quickly moves to another.

DSSS (Direct Sequence Spread Spectrum) spreads each data bit into multiple smaller bits called chips using a spreading code such as the Barker sequence. This spreads the signal across a wider bandwidth, providing resistance to noise.

FHSS spreads the signal by changing frequencies. DSSS spreads the signal by encoding bits into longer chip sequences.

DSSS allowed higher data rates than FHSS and was widely adopted in 802.11b.

3) Modulation Schemes in the PHY Layer

Modulation is the process of converting digital bits into radio signals.

Early Wi-Fi used BPSK and QPSK. As standards evolved, higher-order modulation schemes were introduced such as 16-QAM, 64-QAM, 256-QAM, and 1024-QAM.

BPSK carries 1 bit per symbol and is very robust but slow. QPSK carries 2 bits per symbol. 16-QAM carries 4 bits. 64-QAM carries 6 bits. 256-QAM carries 8 bits. 1024-QAM carries 10 bits per symbol.

Higher QAM increases data rate but requires a cleaner signal (high SNR). That is why devices close to the AP use high QAM while distant devices fall back to lower QAM.

4) Significance of OFDM in WLAN and How It Improves Performance

OFDM (Orthogonal Frequency Division Multiplexing) divides a channel into many small orthogonal subcarriers. Each subcarrier carries a small portion of the data in parallel.

This provides resistance to multipath fading because reflections affect only some subcarriers, not the whole signal.

OFDM greatly improves spectral efficiency and supports high data rates. It is the backbone of 802.11a, g, n, ac, and ax.

Without OFDM, modern high-speed Wi-Fi would not be possible.

5) Wi-Fi Frequency Bands and Channel Division

Wi-Fi primarily operates in three bands: 2.4 GHz, 5 GHz, and 6 GHz.

The 2.4 GHz band is crowded and has overlapping channels. Only channels 1, 6, and 11 do not overlap.

The 5 GHz band offers many more channels and less interference.

The 6 GHz band introduced with Wi-Fi 6E provides very wide clean spectrum, allowing extremely high throughput and less congestion.

Channels can be 20, 40, 80, or 160 MHz wide. Wider channels carry more data but consume more spectrum.

6) Role of Guard Intervals in WLAN Transmission

Guard Interval (GI) is a small time gap between OFDM symbols to prevent Inter Symbol Interference caused by multipath reflections.

The standard GI is 800 nanoseconds. A Short GI of 400 nanoseconds improves efficiency and increases data rate by about 10%.

GI ensures that delayed signals from reflections do not interfere with the next symbol.

7) Structure of an 802.11 PHY Layer Frame

The PHY frame consists of three main parts.

The Preamble is used for synchronization, allowing the receiver to detect the signal and align timing.

The PLCP header contains information about the data rate and length of the frame.

The PSDU contains the actual MAC frame (user data).

The preamble ensures proper detection. The header tells how to decode. The data carries the payload.

8) Difference Between OFDM and OFDMA

OFDM allows only one user to use all subcarriers at a time.

OFDMA divides the subcarriers into resource units and allows multiple users to transmit simultaneously.

OFDM is efficient for single users. OFDMA is efficient for many users in dense environments.

OFDMA is a key feature of 802.11ax.

9) Difference Between MIMO and MU-MIMO

MIMO uses multiple antennas to send multiple spatial streams to a single user, increasing speed.

MU-MIMO allows the AP to send data to multiple users at the same time using different spatial streams.

MIMO improves data rate. MU-MIMO improves network capacity.

10) PPDU, PLCP, and PMD in the PHY Layer

PPDU is the complete PHY frame transmitted over the air.

PLCP is the sublayer that adds headers to the MAC frame before transmission.

PMD is the sublayer responsible for modulation and actual radio transmission.

The flow is: MAC frame → PLCP adds header → forms PPDU → PMD transmits.

11) Types of PPDU Across Wi-Fi Generations

Different Wi-Fi generations have different PPDU formats.

Legacy OFDM PPDU was used in 802.11a/g.

HT PPDU was introduced in 802.11n for MIMO support.

VHT PPDU was introduced in 802.11ac for MU-MIMO and wider channels.

HE PPDU is used in 802.11ax for OFDMA and high efficiency.

Each new PPDU format adds fields to support new technologies.

12) How Data Rate is Calculated in Wi-Fi

Data rate depends on several factors.

It depends on the number of bits per symbol decided by modulation.

It depends on the number of subcarriers in the channel.

It depends on coding rate used for error correction.

It depends on symbol duration which is affected by guard interval.

It also depends on the number of spatial streams in MIMO.

Higher modulation, wider channels, short GI, and more spatial streams all increase data rate.