WiFi Training Program

G ADITHYA PRASAD

Assignment Questions - Module 1

1. In which OSI layer the Wi-Fi standard/protocol fits.

Since Wi-fi is a secure, wireless medium of data transmission, it operates in the Physical and Data-Link Layer of the OSI model.

- In Wi-fi, the data is transmitted over the air using radio waves and several modulation techniques. So, the data transmission part of Wi-fi falls under the physical layer of the OSI model.

- Wi-fi does Frame Formatting, Addressing, Error Detection and Correction (CRC) and uses CSMA/CA for collision avoidance, typically highlighting the MAC sublayer functions of the Data-Link Layer of the OSI model.

- Finally, Wi-fi Does flow control and uses protocols like IP, ARP etc for proper delivery of data which is the LLC sublayer of the data link layer.

2. Can you share the Wi-Fi devices that you are using day to day life, share that device's wireless

capability/properties after connecting to network. Match your device to corresponding Wi-Fi

Generations based on properties.

Some of the Devices I use day-to-day in my life include:

1. Smartphone:

- Standard: Wi-Fi 6 (IEEE802.11ax)

- Frequency: 5 GHz

- Channel width: 20 / 40 / 80 MHz

- Link speed: 25 Mbps – 30 Mbps

- MIMO: 2×2 MIMO

- Security: WPA2 / WPA3

2. Laptop:

- Standard: Wi-Fi 5 (IEEE802.11ac)

- Frequency: 5 GHz (preferred) + 2.4 GHz fallback

- Channel width: up to 80 MHz / 160 MHz

- Link speed: 25 Mbps – 30 Mbps

- MIMO: 2×2 or 3×3

- Modulation: 256-QAM (Wi-Fi 5), 1024-QAM (Wi-Fi 6)

3. Smart TV:

- Standard: Wi-Fi 4 (IEEE802.11n)

- Frequency: Mostly 2.4 GHz

- Channel width: 20 / 40 MHz

- Link speed: 30 Mbps

- MIMO: usually 1×1 or 2×2

3. what is BSS and ESS?

A Basic Service Set consists of one Access point and Multiple client devices. This Wi-fi network is identified by SSID and BSSID. Whereas, Extended Service Set consists of Multiple Aps connected by a wired lan. Every AP is connected to multiple client devices. All the Aps share a common SSID. ESS enables easy roaming without getting disconnected as while moving, the device automatically gets connected to another AP.

4. what are the basic functionalities of Wi-Fi Access point?

Functionalities of AP:

- Converts wired Ethernet → wireless signals

- Creates the Wi-Fi network (SSID)

- Handles Authentication using Security Protocols like WPA2/WPA3.

- Does frame forwarding and bridging by connecting wireless clients with wired switched/ routers.

- Does Media Access control for avoiding collisions using CSMA/CA protocol.

- Offers roaming support by helping devices to switch between Aps in ESS.

- Controls channel parameters like Bandwidth and Data transmission Rate.

5. What are Differences between Bridge mode and Repeater mode.

| Bridge Mode | Repeater Mode |
| --- | --- |
| Connect two Networks | Extend Network Coverage of a given wi-fi network. |
| No clients are connected in this mode of operation. | Clients are present. |
| No major Loss is incured | Loss might occur and re-transmission may be required |
| Point-to-Point Link | Signal Re-Transmission |
| Usually used in a building-to-building link. | Used for eliminating dead zones where signal gets completely lost. |

6) Differences between 802.11a and 802.11b

| Feature | IEEE 802.11a | IEEE 802.11b |
| --- | --- | --- |
| Frequency band | 5 GHz | 2.4 GHz |
| PHY technique | OFDM | DSSS / CCK |
| Max data rate | 54 Mbps | 11 Mbps |
| Interference | Low (cleaner band) | High (Bluetooth, microwave) |
| Range | Shorter | Longer |
| Wall penetration | Poor | Better |
| Channels | Many non-overlapping | Only 3 non-overlapping |
| Cost (early days) | Expensive | Cheaper, popular |
| Adoption | Enterprise | Homes / SOHO |

Key idea:
802.11a = faster but shorter range.
802.11b = slower but longer range.

7) 2.4 GHz vs 5 GHz — What you will observe on your laptop

Set hotspot/AP to 2.4 GHz only, note properties. Repeat for 5 GHz.

Check in Windows:
Wi-Fi → Properties → Link speed / Protocol / Channel

| Property you’ll see | 2.4 GHz Observation | 5 GHz Observation |
| --- | --- | --- |
| Protocol | 802.11n / ax | 802.11ac / ax |
| Link speed | Lower (e.g., 144–300 Mbps) | Higher (e.g., 866 Mbps+) |
| Channel width | 20/40 MHz | 40/80/160 MHz |
| Signal strength | Stronger through walls | Drops faster with walls |
| Interference | High | Low |
| Channel number | 1–13 | 36+ |
| Latency (ping) | Slightly higher | Lower |
| Stability | Good for distance | Best for same room speed |

What you learn:
2.4 GHz = coverage.
5 GHz = speed.

8) Difference between IEEE and WFA

| Aspect | IEEE | Wi-Fi Alliance |
| --- | --- | --- |
| Full form | Institute of Electrical and Electronics Engineers | Wi-Fi Alliance |
| Role | Creates Wi-Fi standards (802.11) | Certifies devices for interoperability |
| Work | Technical PHY/MAC specs | Testing & branding (Wi-Fi 6, 6E, 7) |
| Output | 802.11 documents | “Wi-Fi CERTIFIED” logo |
| Focus | How Wi-Fi works | Devices work together correctly |

IEEE writes the rules. WFA checks devices follow them.

9) Types of Wi-Fi Backhaul (Internet Connectivity to AP/Router)

Backhaul = how your router/AP gets internet.

| Backhaul Type | Description | Properties |
| --- | --- | --- |
| Fiber (FTTH) | Optical fiber to home | Very high speed, low latency |
| DSL | Telephone copper line | Moderate speed |
| Cable | Coaxial cable | Good speed, shared medium |
| Cellular (4G/5G) | SIM-based hotspot | Portable, higher latency |
| Wireless P2P | Dish/antenna from ISP tower | Used in rural areas |
| Ethernet leased line | Dedicated enterprise link | Symmetric, reliable |

Example (typical home/college):

- Likely Fiber (FTTH) from ISP

- Properties: 100–1000 Mbps, low latency, stable

10) Wi-Fi Topologies and Use Cases

| Topology | Description | Use Case |
| --- | --- | --- |
| Infrastructure Mode | Clients connect to AP | Homes, offices |
| Ad-Hoc (IBSS) | Device ↔ Device | File sharing without AP |
| Mesh Topology | APs connect to each other wirelessly | Large homes, campuses |
| Repeater Mode | Extends existing Wi-Fi | Increase coverage |
| Point-to-Point (P2P Bridge) | AP ↔ AP long distance | Building-to-building link |
| Point-to-Multipoint | One AP to many remote sites | WISP deployments |